# Temperature measurement and ON/OFF control

The hardware is based on AT89C51 microcontroller, which is compatible with 51 family micro controllers. The unit has IC Temperature Transducer to sense the different temperatures from room temperature (30 to 100). The transducer is connected to signal conditioning unit. The sensor produces very low current signals (in terms of micro amps), which cannot be measured by ADC. So the weak signal is amplified to 0-5V level by signal conditioner.

Micro

Controller

AT89C51

Signal

Conditioner

IC

Temperature			      		      To

ADC

LCD

Keypad

Transducer		     	     		      Relay

Coils

The output of signal conditioner is given to the Analog to Digital Converter (ADC). The ADC has resolution of 8Bit. The LCD display is used to indicate the temperature. The user can give required value using keypad.

When the temperature exceeds the set point the controller reads the range through ADC and OFF the relay. After the temperature decrease it again ON the relay. So it maintain the temperature range given by the user. Through relay we can connect any device.

The Basic firmware (software for microcontroller) is written in C language and compiled using C compiler (Avocet Make). The compiler produces the final machine code of the program and it is stored in the Flash memory of microcontroller.